This is the continuation of our previous chats/conversations held between 16 to 17 Aug…

NOTE: Some of the conversations we had were repeating some of the previous conversations due to ChatGPT not remembering or recalling them or it would divert away from the standards, procedures,

============

Chat 001-21

============

ChatGPT’s Response:

You're right. I said I would provide it, and I didn't. Here is Stage 2 now.

I am also going to keep this stage deliberately narrow: we are verifying the Windows 11 Pro USB that already exists. We are not booting from it yet, and we are not touching the internal SSD.

STAGE 2 — WINDOWS 11 PRO INSTALLATION USB VERIFICATION

Objective

Before we allow the laptop to boot from the installation media, we need to establish that the USB you already created is:

- the intended Windows 11 installation media,

- configured for Windows 11 Pro,

- using the intended ei.cfg,

- configured for UEFI/GPT,

- and not missing anything required for the next stage.

Your project history establishes that the USB was created with Rufus and that Sources\ei.cfg was added to force the Professional edition. The documented ei.cfg is:

[EditionID]
Professional
[Channel]
Retail
[VL]
0

So we are verifying that existing work, not recreating it.

PART 1 — Connect the Installation USB

Step 1

From the currently running Windows 11 Home factory installation:

- Connect the USB drive that contains the Windows 11 Pro installation media.

- Do not open Windows Setup.

- Do not double-click setup.exe.

- Do not format the USB.

- Do not make any changes to the internal SSD.

PART 2 — Identify the USB

Step 2

Open File Explorer.

Step 3

Select This PC.

Step 4

Identify the USB drive.

You should see a removable USB storage device containing the Windows installation files.

Do not assume its drive letter.

For the remaining instructions below, I'll call it:

USB_INSTALL

If Windows assigned it E:, for example, then USB_INSTALL means E:.

PART 3 — Verify the Windows Installation Structure

Step 5

Open the USB.

At the root level, you should see the normal Windows installation structure.

Among the items we care about, there should be:

boot
efi
sources
support
bootmgr
bootmgr.efi
setup.exe

There may be additional files/directories. That's fine.

Step 6

Open:

USB_INSTALL\sources

Step 7

Locate:

ei.cfg

STOP POINT A

Do not modify anything.

Open ei.cfg with Notepad.

It should contain exactly:

[EditionID]
Professional
[Channel]
Retail
[VL]
0

What I need from you

Tell me whether the file contains those four lines.

Do not send me your Windows product key.

PART 4 — Verify the Installation Edition Information

Step 8

Remain in:

USB_INSTALL\sources

Look for:

install.wim

or:

install.esd

There should be one of those installation-image files.

Step 9

Do not attempt to modify or replace it.

At this point we're only establishing which installation-image format the USB uses.

What I need from you

Tell me whether you have:

- install.wim

- install.esd

- or neither.

PART 5 — Verify the USB's Boot Structure

Step 10

Return to the USB root.

Confirm that these exist:

\EFI
\EFI\BOOT

Step 11

Open:

USB_INSTALL\EFI\BOOT

There should be a Windows UEFI bootloader there, normally:

BOOTX64.EFI

STOP POINT B

Do not change anything.

Tell me whether BOOTX64.EFI exists.

PART 6 — Verify the USB File System / Partition Configuration

This part is important because Rufus created the USB.

Step 12

Right-click the USB drive in File Explorer.

Select:

Properties

Step 13

Record:

- File system

- Used space

- Free space

- Capacity

Do not change anything.

Step 14

Now open:

Disk Management

You can do that by:

- Press Win + X.

- Select Disk Management.

Step 15

Locate the USB by its capacity.

Be extremely careful here.

You should see:

- the internal ~1 TB SSD

- the external preservation drive, if connected

- the Windows installation USB.

STOP POINT C

Do not right-click, delete, format, initialize, convert, shrink, extend, or otherwise modify any disk.

We are only identifying the installation USB.

Step 16

Once you've identified the USB, tell me:

- Disk number

- approximate capacity

- partition layout shown by Disk Management

- filesystem shown, if displayed.

PART 7 — Verify Rufus Configuration

We don't need to recreate the USB, but I want to establish what Rufus actually configured.

If you still have the Rufus-created USB and Rufus is still available:

Step 17

Do not click Start.

Launch Rufus.

Step 18

Select the Windows 11 Pro installation USB as the Device.

Step 19

Verify the displayed configuration.

We want to see:

Partition scheme:

GPT

Target system:

UEFI

The exact remaining fields can vary depending upon Rufus version and Windows ISO.

CRITICAL

Do not press START.

We are inspecting the existing USB only.

Close Rufus when finished.

PART 8 — Verify That We Are Not Accidentally Preparing the Wrong Drive

This is a safety check that I want explicitly documented before we proceed.

At this point you should have three potentially relevant storage devices:

A — Internal ASUS SSD

Approximately:

953.86 GB

This is the drive that eventually gets erased.

B — Factory Macrium image storage

Your external USB drive containing the factory image.

C — Windows 11 Pro installation USB

The USB we are currently verifying.

These three drives must remain distinctly identifiable.

The installation USB must never be confused with the factory-image drive.

And neither external drive is to be selected as the Windows installation target.

PART 9 — What I Need You to Report Back

Rather than having you upload anything, give me the following textual results:

STAGE 2 — USB VERIFICATION

1. USB drive letter:
2. USB capacity:
3. USB filesystem:
4. ei.cfg present: YES / NO
5. ei.cfg contents:
6. install.wim or install.esd:
7. EFI\BOOT\BOOTX64.EFI present: YES / NO
8. Disk Management disk number:
9. Disk Management approximate capacity:
10. Rufus Partition Scheme:
11. Rufus Target System:

For #5, you can simply say:

Matches exactly.

if it contains the four lines I gave you.

Do not provide your Windows Pro product key.

What happens after you report those results

I will evaluate them and give you one of two decisions:

STAGE 2 — PASS

If everything is correct, we proceed to:

STAGE 3 — FINAL PRE-BOOT SAFETY CHECK

That will be the last non-destructive stage before we boot the laptop from the Windows 11 Pro USB.

Or:

STAGE 2 — NOT PASS

I'll identify exactly what needs correcting without having you recreate the USB unnecessarily.

One very important boundary

Do not boot the USB yet.

Don't enter Windows Setup.

Don't delete anything.

Don't run diskpart clean.

Don't format the internal SSD.

Don't alter BIOS/UEFI Secure Boot.

Don't clear TPM.

Don't change the Secure Boot keys.

Don't modify the factory partitions.

Stage 2 is strictly verification of the installation media.

Once you give me the eleven results above, I will make the Stage 2 PASS/FAIL determination and then give you Stage 3.

My Response: Answers will be in the Current Project Status 03  located in the GitHub repository.

============

Chat 001-22

============

ChatGPT’s Response:

STAGE 3 — FINAL PRE-BOOT SAFETY CHECK

This is the last stage before we actually boot the ASUS from the Windows 11 Pro USB.

The purpose is not to collect more factory information. It is to make sure that when we leave the existing Windows installation, we have everything necessary to safely reach the Windows Setup partition-selection screen.

Part 1 — Confirm the two-drive installation environment

You have already established that only these two storage devices will be connected during installation:

Drive A — Internal ASUS SSD

Approximately 953.86 GB.

This is the ONLY drive that will eventually be erased.

Drive C — Windows 11 Pro Installation USB

Approximately 28.9 GB.

This is the ONLY removable drive that will be used to boot Windows Setup.

Required condition

The Macrium factory-image drive remains physically disconnected.

Do not reconnect it until I specifically tell you to.

Part 2 — Verify the factory-image safeguard

Before shutting down the factory Windows installation:

- Confirm that the external USB containing the Macrium factory image is disconnected.

- Confirm that you know where that drive is stored.

- Do not connect it during Windows installation.

We already established that the factory image exists and contains the factory SSD structure.

We're not going to expose that drive to Windows Setup.

Part 3 — Verify the Windows 11 Pro USB one final time

Leave the Windows 11 Pro installation USB connected.

Do not open setup.exe from within Windows.

We're going to boot from it.

Part 4 — Do NOT change the Secure Boot configuration

This is particularly important given what we discovered during the factory-state investigation.

Your documented Secure Boot state includes:

- PK — Factory

- KEK — Factory

- db — Factory

- dbx — Modified

- dbt — Factory

Do not:

- Reset to Setup Mode

- Clear Secure Boot keys

- Export/import keys

- Disable Secure Boot

- Clear TPM

- change Secure Boot key databases.

We are installing Windows 11 Pro using the existing UEFI/Secure Boot environment.

Part 5 — Shut down Windows

From the factory Windows 11 Home desktop:

- Save anything that happens to be open.

- Close all applications.

- Confirm the Macrium-image USB is disconnected.

- Confirm the Windows 11 Pro installation USB is connected.

- Perform a normal Shut down.

Do not restart into Windows.

Part 6 — Enter the ASUS boot-selection environment

With the laptop powered completely off:

- Press the laptop's power button.

- Immediately begin repeatedly pressing F8.

On ASUS systems this should bring up the boot-device selection menu.

Do not select anything yet if you are uncertain.

We want to identify the USB by its name/UEFI entry.

Part 7 — Identify the Windows installation USB

The boot menu should show the available boot devices.

We are looking for the UEFI boot entry corresponding to the Windows installation USB.

It may appear as something similar to:

UEFI: [USB manufacturer/model]

or another UEFI USB entry.

Do NOT select:

- Windows Boot Manager

- the internal SSD

- any entry obviously associated with the internal NVMe drive.

Select:

The UEFI entry corresponding to the Windows 11 Pro installation USB.

Then press Enter.

Part 8 — Windows Setup should appear

After booting the USB, Windows Setup should load.

You should eventually reach the Windows Setup screen beginning the installation process.

At this point:

DO NOT delete anything.

DO NOT format anything.

DO NOT open Command Prompt.

DO NOT use DiskPart.

THE CRITICAL CHECKPOINT

Eventually you will reach:

Where do you want to install Windows 11?

This is where STAGE 3 ends and STAGE 4 begins.

STOP THERE.

Do not click:

Delete

Do not click:

Format

Do not click:

New

Do not click:

Load driver

Do not click:

Next

What I want you to report back

At that screen, give me the exact list Windows Setup displays, including:

- Disk number

- partition number

- partition name/type

- size

- free space

For example, if Setup displays something like:

Drive 0 Partition 1 ...

Drive 0 Partition 2 ...

Drive 0 Partition 3 ...

...

write down exactly what it says.

This is our final destructive-operation checkpoint.

I will compare the Windows Setup partition map against the factory layout we've already established.

Only after I confirm that the correct internal SSD is identified will I give you the next stage:

STAGE 4 — DELETING THE FACTORY PARTITIONS

And that is where we will proceed one partition at a time, with you reporting the screen back before the destructive action.

So, for now:

STAGE 2 = PASS.

You are authorized to proceed through Stage 3 up to the Windows Setup "Where do you want to install Windows?" screen.

Stop there and report the partition table to me.

============

Chat 001-23

============

My Statement:

Okay once I get to Part 8 - Windows Setup should appear and we get to the "Where do you want to install Windows 11?" option we have the following...

- Disk 0 Partition 1 ; 260 MB for both Total and Free Space ; Type: System

- Disk 0 Partition 2 ; 16.0 MB for both Total and Free Space ; Type: MSR (Reserved)

- Disk 0 Partition 3 ; 924.5 GB for both Total and Free Space ; Type: Primary

- Disk 0 Partition 4 ; 896.0 MB for both Total and Free Space ; Type: Recovery

- Disk 0 Partition 5: RESTORE ; 28.0 GB Total Size and 6.4 GB Free Space ; Type: Primary

- Disk 0 Partition 6 ; 260.0 MB for both Total and Free Space ; Type: Recovery

- Disk 1 Partition 1: Win11_Installer ; 28.9 Total Size and 21.0 GB Free Space ; Type: Primary

- Disk 1 Partition 2: RUFUS_BOOT ; 1.0 MB Total Size and 30.0 KB Free Space ; Type: Primary

Directly under that window there is a WARNING: Windows 11 can't be installed on disk 0 partition 3 (Show details). After clicking on the "show details" a pop-up window appears. The "Windows 11 Setup" pop-up window says "Installation is not supported for the file system on the current partition. BitLocker Drive Encryption is enabled on the selected partition. Suspend (also referred to as disable) BitLocker in the Control Panel, and then restart the installation"

Is it talking about the "Control Panel" within the Windows 11?

ChatGPT’s Response:

Yes — the message is referring to the Control Panel of the existing Windows installation on Disk 0, but that is NOT what we should do in our situation.

And the partition information you just provided gives us an important confirmation.

First: Disk 0 is confirmed

Your Windows Setup screen now shows:

| Disk 0 | Size | Type | Factory correlation |
| --- | --- | --- | --- |
| Partition 1 | 260 MB | System | EFI/System |
| Partition 2 | 16 MB | MSR | Microsoft Reserved |
| Partition 3 | 924.5 GB | Primary | Factory Windows/OS — C: |
| Partition 4 | 896 MB | Recovery | Windows Recovery |
| Partition 5 — RESTORE | 28 GB | Primary | ASUS RESTORE |
| Partition 6 | 260 MB | Recovery | ASUS/MYASUS recovery |

And:

| Disk 1 | Size | Type |
| --- | --- | --- |
| Partition 1 — Win11_Installer | 28.9 GB | Primary |
| Partition 2 — RUFUS_BOOT | 1 MB | Primary |

This is exactly the distinction we needed to establish.

Disk 0 = internal ASUS SSD.

Disk 1 = Windows 11 Pro installation USB.

That means our biggest installation risk—confusing the installation USB with the internal SSD—is resolved.

Now about that BitLocker warning

Windows Setup says:

"Installation is not supported for the file system on the current partition. BitLocker Drive Encryption is enabled on the selected partition. Suspend (also referred to as disable) BitLocker in the Control Panel, and then restart the installation."

Yes, "Control Panel" means the Control Panel of the Windows installation currently associated with that BitLocker-protected partition.

In our case, that's the factory Windows 11 Home installation on Disk 0 Partition 3.

If we were trying to keep that Windows installation and install/upgrade Windows onto that same encrypted partition, then we would have to deal with BitLocker appropriately.

But we're not doing that.

We are performing a clean installation.

Our objective is to eliminate:

- the factory Windows 11 Home installation,

- its BitLocker-protected OS partition,

- the factory partition structure,

and replace the entire internal SSD with a new Windows 11 Pro installation.

Therefore:

DO NOT BOOT BACK INTO FACTORY WINDOWS TO SUSPEND BITLOCKER.

DO NOT OPEN CONTROL PANEL.

DO NOT DISABLE BITLOCKER.

DO NOT ATTEMPT TO INSTALL WINDOWS ON PARTITION 3.

That warning is actually useful because it confirms what we already knew from Macrium:

Disk 0 Partition 3 is the BitLocker-protected factory OS partition.

We have now reached the critical destructive-operation checkpoint

There is one more important point before I give you the deletion procedure.

Your complete partition table has now matched our preserved factory-state information:

Factory state:

Partition 1 — EFI/System       260 MB
Partition 2 — MSR               16 MB
Partition 3 — OS              924.5 GB
Partition 4 — Recovery         896 MB
Partition 5 — RESTORE           28 GB
Partition 6 — Recovery          260 MB

Windows Setup currently sees:

Disk 0 Partition 1 — System       260 MB
Disk 0 Partition 2 — MSR           16 MB
Disk 0 Partition 3 — Primary      924.5 GB
Disk 0 Partition 4 — Recovery      896 MB
Disk 0 Partition 5 — RESTORE        28 GB
Disk 0 Partition 6 — Recovery       260 MB

That's an excellent match.

And Disk 1 independently identifies itself as:

Win11_Installer
RUFUS_BOOT

So we have positively identified both drives.

STAGE 4 — FACTORY PARTITION REMOVAL

We are now at the point where the destructive operation can occur.

But I'm going to keep this exactly as we established earlier:

We will NOT use DiskPart.

We will NOT run clean.

We will NOT manually create partitions.

We will use Windows Setup's own Delete function.

That lets Windows Setup create the correct GPT/UEFI partition structure automatically when we begin the Pro installation.

Before you click anything

I want you to verify these three things one final time:

1. Disk 0

924.5 GB Partition 3 is the large factory OS partition.

2. Disk 1

28.9 GB Win11_Installer is the installation USB.

3. Factory Macrium image

The external USB containing the factory image is physically disconnected.

You have already told me #3 is true.

If those three conditions are true, we are ready.

The deletion sequence

On the "Where do you want to install Windows 11?" screen:

Step 1

Select:

Disk 0 Partition 6

Do not select Disk 1.

Click:

Delete

Windows Setup will warn you that the partition may contain important files.

Confirm the deletion.

Step 2

Select:

Disk 0 Partition 5: RESTORE

Click:

Delete

Confirm.

Step 3

Select:

Disk 0 Partition 4

Click:

Delete

Confirm.

Step 4

Select:

Disk 0 Partition 3

This is the 924.5 GB BitLocker partition.

Click:

Delete

This is the one that generated the BitLocker warning.

That warning does not mean we should go back to factory Windows.

We are deleting the partition.

If Windows Setup permits the deletion, confirm it.

Step 5

Select:

Disk 0 Partition 2

Click:

Delete

Confirm.

Step 6

Select:

Disk 0 Partition 1

Click:

Delete

Confirm.

STOP AFTER THE LAST DELETION

Do not click Next yet.

We want the screen to show something essentially equivalent to:

Drive 0 Unallocated Space — approximately 953 GB

while Disk 1 continues to show:

Win11_Installer — 28.9 GB

and:

RUFUS_BOOT — 1 MB

That is our next checkpoint.

One important exception

If Windows Setup refuses to delete Partition 3 because of the BitLocker state, STOP.

Do not start experimenting.

Do not go to Control Panel.

Do not reboot.

Do not use diskpart.

Tell me exactly what Windows Setup says.

We'll deal with that specific condition before proceeding.

So the answer to your direct question is:

Yes, the "Control Panel" mentioned in the warning refers to the existing Windows installation's Control Panel.

No, we should not use it.

That instruction is applicable when someone wants to retain the BitLocker-protected Windows installation. We are intentionally destroying that installation as part of this clean installation.

You have now completed the identification checkpoint successfully.

Stage 3 — FINAL PRE-BOOT SAFETY CHECK: PASS.

Stage 4 — FACTORY PARTITION REMOVAL: READY TO EXECUTE.

Start with Disk 0 Partition 6, and proceed one partition at a time as listed above. Stop immediately if Partition 3 refuses to delete or anything differs from the expected behavior.

============

Chat 001-23

============

ChatGPT’s Response:

Excellent. That is exactly the checkpoint we wanted to reach.

You have now successfully completed the destructive portion of the factory-to-Pro transition.

STAGE 4 — FACTORY PARTITION REMOVAL: PASS

The internal SSD is now showing:

Drive 0 — Unallocated Space — approximately 953 GB

And the Windows 11 Pro installation USB remains:

Drive 1 — Win11_Installer — 28.9 GB

with its small RUFUS_BOOT partition.

What this confirms

The following factory partitions have been successfully removed:

- EFI/System — 260 MB

- MSR — 16 MB

- Factory Windows OS — 924.5 GB

- Windows Recovery — 896 MB

- ASUS RESTORE — 28 GB

- ASUS/MYASUS Recovery — 260 MB

The factory BitLocker-protected Windows installation is therefore gone, exactly as intended.

We did not:

- alter Secure Boot keys

- reset Secure Boot

- clear TPM

- disable factory security infrastructure

- use DiskPart

- modify the installation USB

- touch the Macrium backup drive.

And importantly, the BitLocker warning did not prevent the clean installation because we deleted the encrypted partition rather than attempting to install into it.

STAGE 5 — WINDOWS 11 PRO INSTALLATION

This is the next step.

We are now going to let Windows Setup create the new GPT/UEFI partition structure automatically.

Before proceeding

You should currently be on:

Windows Setup → Where do you want to install Windows 11?

with:

Drive 0 — Unallocated Space — approximately 953 GB

selected.

Step 1

Click:

Drive 0 Unallocated Space

Make sure it is highlighted.

Step 2

Click:

Next

Step 3

Do not manually create any partitions.

Do not click:

- New

- Format

- Delete

- Load Driver

- Refresh

Windows Setup should automatically create the required partitions.

You should see Windows Setup create a new partition structure on Drive 0, typically including the required:

- EFI/System partition

- MSR

- Windows/Primary partition

- Recovery partition

The exact sizes displayed by Setup may differ from the factory layout. That is expected.

Step 4

Windows Setup should then begin:

Installing Windows

with stages such as:

- Copying Windows files

- Getting files ready for installation

- Installing features

- Installing updates

- Finishing up

The computer will restart during the process.

⚠️ ONE CRITICAL POINT DURING THE RESTART

When the computer restarts, do not accidentally boot back into the USB installation process from the beginning.

If you see a message such as:

Press any key to boot from USB...

do not press a key.

Allow the system to continue from the newly created internal Windows installation.

If the system instead returns to the beginning of Windows Setup, stop and tell me exactly what you see.

Our next checkpoint

I don't want to combine the installation and post-installation configuration into one giant procedure.

Your next check-in should be after Windows Setup has completed the installation and you arrive at the Windows 11 initial setup/OOBE screens.

At that point, do not start installing ASUS software or drivers yet.

We'll establish the clean Windows 11 Pro baseline first.

Current project state

Factory Preservation: ✅ COMPLETE

Stage 2 — USB Verification: ✅ PASS

Stage 3 — Final Pre-Boot Safety: ✅ PASS

Stage 4 — Factory Partition Removal: ✅ PASS

Stage 5 — Windows 11 Pro Installation: READY

You have reached the point we've been working toward since the beginning:

The factory Windows 11 Home environment is gone, the factory SSD has been cleared, and the machine is ready to become the clean Windows 11 Pro installation we designed.

Go ahead and select Drive 0 Unallocated Space → Next.